Denon DN-V755 Firmware Installation Instructions

v.0.39

- Extract the update file folder.

- You will obtain the .ISO file after extracting.

- Drag and drop the .ISO file to the Software folder on the DN-V755.

- Once the file is done transferring, cycle the power on the DN-V755.

The machine is now updated.

Firmware Update Details/Notes – Version 0.39

Added the new functions below:

- Password function was added for CIFS and browser access.

- Automatic file download function was added. Files can be automatically downloaded from the preset FTP server.

- On boot up with DIP switch #1 on the back panel ON, unit will boot up without a password.

- On boot up with DIP switch #2 on the back panel ON, unit will boot up without starting playback of the preset playlist.

Fixed the issue below;

- On the network with a DHCP server and without a WINS server installed, if a static IP is assigned, sometimes unit will lock up.

Please note, you can check the firmware version currently installed on your DN-V755 by accessing the settings tab in the GUI or via RS232.